|  | InterSense Inc. 36 Crosby Drive, Suite 150 Bedford, MA 01730 |
| --- | --- |

Procedure: Upgrading IS-900 Processor Firmware

Resources Required

Windows PC

Null modem RS-232 cable, InterSense P/N 078-00001-0000

IS-900 Processor with power cable

CD-ROM containing upgrade Zip file (intrack42007.zip)

Procedure

1.	Connect the processor to COM1 of your Windows PC. If you need to use a port other than COM1, change the "-com 1" switch setting in "upgrade.bat" to the appropriate port before running the batch file in step 3.

2.	Put the processor into monitor mode.

a.	If the processor is off, power it up and press F1 on the front panel as the row of dots appears on the LCD.

b.	If the processor is already running, navigate the LCD menu using the buttons on the front panel of the processor: System Config -> Monitor -> Yes

c.	The LCD should read Monitor Mode.

3.	Extract the Zip file into a directory on your Windows PC. From that directory, run "upgrade.bat" to begin the file transfer. Wait for the transfer to finish.

4.	Turn processor power off and then on again. Verify that processor initializes successfully and without errors.